| - Identification |  |
| --- | --- |
| - 1.1 Product name： | PEA-4139 |
| Chemicals： | Waterborne hydroxyl polyester-acrylic dispersion |
| Product classification:： | Fatty-acid modified waterborne hydroxyl polyester-acrylic emulution |
| - 1.2 Product use suggestions： | Coating |
| - 1.3 Supplier information： |  |
| Name： | Yingde Guocai Fine Chemical Co., Ltd |
| Address： | Kaidi Industrial Park, genggukeng, Taiping Village, Baisha Town, Yingde City, Guangdong Province |
| Tel： | 86-763-2811205 |
| Fax： | 86-763-2811024 |

| - Hazards Identification |  |
| --- | --- |
| - 2.1 GHS classification: | Flammable liquids, Category 4 (H227) |
| - 2.2 GHS label elements: |  |
| - Pictogram: | None |
| - 2.3 Signal word： | Warning |
| - 2.4 Hazard statements: | H227 Combustible liquid. H316 Causes mild skin irritation. H320 Causes eye irritation. H332 Harmful if inhaled. H360 May damage fertility or the unborn child. H370 Causes damage to organs. H373 May cause damage to organs (liver) through prolonged or repeated exposure. |
| - 2.5 Precautionary statements: | P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P280 Wear protective gloves/eye protection/face protection. P370+P378 In case of fire: Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide to extinguish. P403+P235 Store in a well-ventilated place. Keep cool. P501 Dispose of contents/container in accordance with local regulations. |
| - 2.6 Other hazards: | Spray application may generate mist or aerosol; avoid inhalation. Eye contact may cause temporary irritation or discomfort. |

| - Composition/Information on Ingredients |  |  |
| --- | --- | --- |
| - Product type: | Mixture |  |
| - Components |  |  |
| Chemical name | CAS No. | Content (% w/w) |
| Acrylic copolymer | Trade secret | > 40 |
| Deionized water | 7732-18-5 | > 45 |
| Diethylene glycol monobutyl ether | 112-34-5 | <3 |

| - First aid measures |  |
| --- | --- |
| - 4.1 General measures: | Remove all contaminated clothing immediately. |
| - 4.2 Ingestion: | If swallowed, do not induce vomiting. Seek medical attention immediately. Never give anything by mouth to an unconscious person. |
| - 4.3 Eye contact: | Immediately hold the eyelids open and rinse with plenty of gently flowing water for at least 20 minutes. Tilt the head to prevent contaminated water from entering the unaffected eye. Seek medical attention immediately. |
| - 4.4 Skin contact: | Wash immediately with soap and plenty of water. Seek medical attention if a skin reaction occurs. |
| - 4.5 Inhalation: | If inhaled and feeling unwell, seek medical attention. |

| - Fire fighting measures |  |
| --- | --- |
| - 5.1 Suitable extinguishing media: | Carbon dioxide (CO2), alcohol-resistant foam and dry chemical. Use water spray for large fires. |
| - 5.2 Unsuitable extinguishing media: | High-volume water jet. |
| - 5.3 Specific hazards: | Combustion may release carbon monoxide, carbon dioxide, nitrogen oxides and other fumes. Do not inhale smoke or combustion products. |
| - 5.4 Firefighter protection: | Wear self-contained breathing apparatus. Prevent contaminated fire-fighting water from entering soil, groundwater or surface water. |

| - Measures for accidental leakage |  |
| --- | --- |
| - 6.1 Personal precautions and emergency procedures: | Wear appropriate protective equipment. Ensure adequate ventilation/exhaust. Keep unauthorized personnel away. |
| 6.2 Environmental precautions: | Do not discharge into sewers, wastewater or soil. |
| 6.3 Containment and cleanup: | Absorb with suitable chemical absorbent or dry sand, if necessary, and collect in a closed container. |

| - Operation and storage |  |
| --- | --- |
| - Safe handling: | Use with adequate exhaust ventilation. Avoid contact with skin and eyes. Do not breathe vapour or spray. Keep away from food, drink and tobacco. Wash hands before breaks and after work. Remove contaminated or soaked clothing. |
| - Safe storage: | Keep container tightly closed in a cool, well-ventilated place. Follow the storage conditions specified in the product information sheet to maintain product quality. |

| - Exposure control / personal protection |  |
| --- | --- |
| 8.1 Exposure controls: |  |
| Respiratory protection: | Use suitable respiratory protection during spray application. |
| Hand protection： 喷涂过程中要求有呼吸防护设备。 | Wear protective gloves. |
| Suitable glove materials: | EN 374-3 |
| Fluoroelastomer (FKM): Thickness ≥ 0.4 mm; breakthrough time ≥ 480 min. |  |
| Butyl rubber (IIR): Thickness ≥ 0.5 mm; breakthrough time ≥ 480 min. |  |
| Nitrile rubber (NBR): Thickness ≥ 0.35 mm; breakthrough time ≥ 480 min. |  |
| Recommendation: | Discard contaminated gloves. |
| Eye protection: | Wear safety goggles or a face shield. |
| Body protection: | Wear suitable protective clothing. |

| - Physical and chemical properties |  |
| --- | --- |
| 9.1 Appearance: | Milky-white liquid |
| 9.2 Odour threshold: | No data available |
| 9.3 pH (1% aqueous solution): | 7-9 |
| 9.4 Ionic character: | Non-ionic |
| 9.5 Initial boiling point: | Approx. 100°C |
| 9.6 Flash point: | 76°C |
| 9.7 Evaporation rate: | No data available |
| 9.8 Flammability (solid, gas): | Not applicable |
| 9.9 Heat of combustion: | Not applicable |
| 9.10 Vapour pressure: | Approx. 130 hPa at 50°C |
| 9.11 Relative vapour density: | No data available |
| 9.12 Density: | Approx. 1.06 g/cm³ |
| 9.13 Water solubility: | Completely miscible |
| 9.14 Surface tension: | No data available |
| 9.15 Log Pow: | No data available |
| 9.16 Auto-ignition temperature: | Not applicable |
| 9.17 Ignition temperature: | No data available |
| 9.18 Decomposition temperature: | No data available |
| 9.19 Dynamic viscosity: | <400 mPa·s |
| 9.20 Explosive properties: | No data available |
| 9.21 Dust explosion class: | Not applicable |
| 9.22 Solids content: | 47 ± 2% (150°C, 30 min) |
| 9.23 Other information: | These values are not product specifications. Refer to the product technical data sheet. |

| - Stability and reactivity |  |
| --- | --- |
| 10.1 Chemical stability: | Stable under recommended storage and handling conditions. Avoid freezing, overheating and prolonged sunlight. |
| 10.2 Hazardous decomposition products: | Thermal decomposition or combustion may produce carbon monoxide, carbon dioxide, nitrogen oxides and irritating fumes. |
| 10.3 Possibility of hazardous reactions: | No known hazardous reactions under normal conditions of use. Avoid contact with strong oxidizing agents, strong acids and strong bases. |

| - Toxicity information |  |
| --- | --- |
| No toxicological studies are available for this product. |  |
| The following reference data are for diethylene glycol monobutyl ether (CAS No. 112-34-5): |  |
| 11.1 Acute toxicity: | Oral LD50 (rat): approx. 3,306 mg/kg. Inhalation LC50 (rat, 4 h): >1.5 mg/L (aerosol). Dermal LD50 (rabbit): 2,764 mg/kg. Based on available data, the substance does not meet the classification criteria for acute oral, dermal or aerosol inhalation toxicity. |
| 11.2 Skin irritation: | Species: Rabbit. Classification: Not classified for skin corrosion/irritation. Result: Prolonged or repeated contact may cause skin dryness. |
| 11.3 Eye irritation: | Species: Rabbit. Classification: Eye irritant. Result: Undiluted diethylene glycol monobutyl ether may cause marked eye irritation. |
| 11.4 Sensitization: | Species: Guinea pig. Classification: Not classified as a skin sensitizer. Result: No allergic skin reaction was observed in animal studies. |
| 11.5 Mutagenicity: | Most bacterial reverse mutation and chromosome aberration tests were negative. Some in-vitro gene mutation test results were inconsistent. |
| 11.6 Carcinogenicity: | No carcinogenic effects were observed in animal studies. |
| 11.7 Reproductive toxicity: | Based on available animal studies, diethylene glycol monobutyl ether showed no significant effects on fertility, teratogenicity or developmental toxicity. |
| 11.8 Specific target organ toxicity (single/repeated exposure): | Based on available data, the classification criteria for specific target organ toxicity - single exposure are not met. |
| 11.9 Aspiration hazard: | No data available. |
| 11.10 Additional information: | Eye contact may cause marked irritation, redness and pain. Prolonged or repeated skin contact may cause mild irritation, redness or dryness. Vapour pressure is low at normal temperatures; a single vapour exposure is not expected to cause significant adverse effects. Maintain good ventilation and avoid inhalation during spraying, heating or aerosol generation. Swallowing large amounts may cause discomfort. |

| - Ecological information |  |
| --- | --- |
| No ecotoxicological studies are available for this product. |  |
| The following reference data are for diethylene glycol monobutyl ether (CAS No. 112-34-5): |  |
| 12.1 Ecotoxicity - fish: | - LC50 (96 h), bluegill sunfish: 1,300 mg/L. |
| 12.1 Ecotoxicity - other: | - EbC50 (96 h), green algae: >100 mg/L. EC50 (48 h), Daphnia magna: >100 mg/L. |
| 12.2 Persistence and degradability: | - Biodegradation: 89-93% in 28 days. Test method: OECD 301C or equivalent. |
| 12.3 Other adverse effects: | - No data available. |

| - Handling precautions |  |
| --- | --- |
| Dispose of waste in accordance with applicable national and local regulations. Within the EU, disposal should follow the applicable European Waste Catalogue (EWC) requirements. |  |
| Disposal methods: | Empty containers as completely as possible, e.g. by draining, scraping or pouring until drip-free. Send to an appropriate collection or recycling facility in accordance with chemical-industry recovery schemes. Recycle containers in accordance with national and environmental regulations. Do not discharge waste through wastewater. |

| - Transportation information |  |
| --- | --- |
| 14.1 Road and rail transport: | Not regulated as dangerous goods for land transport. |
| 14.2 Sea transport: | Not regulated as dangerous goods for sea transport. |
| 14.3 Air transport: | Not regulated as dangerous goods for air transport. |
| 14.4 Special precautions: | Non-dangerous goods. Maintain temperature between 5°C and 35°C. Keep away from food, acids and bases. |

- Regulatory information

Relevant safety, health and environmental regulations for the substance or mixture:

Complies with the following regulatory requirements:

Regulations on the Safety Management of Hazardous Chemicals, State Council Order No. 591

GB/T 16483 Safety Data Sheet for Chemical Products - Content and Order of Sections

GB 13690 General Rule for Classification and Hazard Communication of Chemicals

GB 30000.2-29 Rules for Classification and Labelling of Chemicals

GB 15258 General Rules for Preparation of Precautionary Labels for Chemicals

- Other information

To the best of our knowledge, the information herein is accurate as of the revision date. It is intended solely as guidance for safe handling, use, processing, storage, transport, disposal and release, and does not constitute a warranty or quality specification. The information applies only to the specified material and may not be valid when used with other materials or in other processes unless specifically stated otherwise.